|  | Simulation of Part1 Date: 23 Temmuz 2018 Pazartesi Designer: Solidworks Study name: Thermal 1 Analysis type: Thermal(Steady state). Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Thermal Loads 4 Contact Information 5 Mesh information 6 Sensor Details 7 Study Results 8 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Part1 Current Configuration: Varsayılan. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split2[2]; Solid Body; Mass:0.433768 kg Volume:0.000160655 m^3 Density:2700 kg/m^3 Weight:4.25093 N; C:\Users\nailt\Desktop\my_git\Thermal-Analysis\Part1.SLDPRT Jul 23 14:35:35 2018. Split2[1]; Solid Body; Mass:0.000612 kg Volume:6e-007 m^3 Density:1020 kg/m^3 Weight:0.0059976 N; C:\Users\nailt\Desktop\my_git\Thermal-Analysis\Part1.SLDPRT Jul 23 14:35:35 2018.

# Study Properties Study name; Thermal 1. Analysis type; Thermal(Steady state). Mesh type; Solid Mesh. Solver type; FFEPlus. Solution type; Steady state. Contact resistance defined?; No. Result folder; SOLIDWORKS document (C:\Users\nailt\Desktop\my_git\Thermal-Analysis).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/mm^2 (MPa).

# Material Properties Model Reference; Properties; Components. Name:; 1060 Alloy. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Thermal conductivity:; 2 W/(cm.K). Specific heat:; 215.105 Cal/(kg.C). Mass density:; 2.7 g/cm^3.; SolidBody 1(Split2[2])(Part1). Curve Data:N/A. Name:; Thermal-Paste. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Thermal conductivity:; 0.28 W/(cm.K). Specific heat:; 331.262 Cal/(kg.C). Mass density:; 1.02 g/cm^3.; SolidBody 3(Split2[1])(Part1). Curve Data:N/A.

# Thermal Loads Load name; Load Image; Load Details. Heat Power-1; Entities:; 1 face(s). Heat Power Value:; 25 W.. Convection-1; Entities:; 47 face(s). Convection Coefficient:; 5 W/(m^2.K). Time variation:; Off. Temperature variation:; Off. Bulk Ambient Temperature:; 300 Kelvin. Time variation:; Off..

# Contact Information Contact; Contact Image; Contact Properties. Global Contact; Type:; Bonded. Components:; 1 component(s). Options:; Compatible mesh..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.00726609 m. Tolerance; 0.000363304 m. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 17604. Total Elements; 8728. Maximum Aspect Ratio; 20.228. % of elements with Aspect Ratio < 3; 17.6. % of elements with Aspect Ratio > 10; 0.458. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:03. Computer name:.

# Sensor Details No Data

# Study Results Name; Type; Min; Max. Thermal1; TEMP: Temperature; 3.379e+002Kelvin Node: 248; 3.451e+002Kelvin Node: 17454. Part1-Thermal 1-Thermal-Thermal1. Image-1.

# Conclusion